# Methodology

This chapter describes the methodology used to simulate a real environment. It includes the models and equations used to describe the different components. The different algorithms are also described in this chapter in order of intelligence levels.

## System Components

### Office Building Model

The office building is modeled first by its energy consumption. The stationary battery of the building is also represented by a battery model. The solar panels are represented by a solar panel model, and the grid is represented by a grid model.

#### Energy Consumption

For the building’s energy consumption Cb(t), we utilize the ELMAS dataset (European Load Model for Aggregated Sectors), specifically the Office cluster (#5). Unfortunately, the dataset only contains mean data for the classes of the cluster, so we calculated a sample building’s annual energy. We then extracted this data into the Time_series_18_clusters.csv file. The following processing steps were followed:

Let Lt be the mean hourly energy load of the different classes of the Office cluster (T=8,736 hours for the year 2018) with yearly energy:

Ecluster=t=1TLt

To be able to represent a single office building, we calculate the per-customer annual energy for each class c belonging to the Office cluster column:

ec=EcNc

where Ec is the annual energy and Nc the number of customers in class c. The 75th percentile is used to scale the energy to represent a medium to big office.

$$\begin{equation}
    E_{target} = Q_{0.75}\!\left(\{e_c\}\right) = 58{,}831 \; \text{\kwh}
    \end{equation}$$

The energy profile Lt is then scaled using the energy target of the single building:

Ltscaled=Lt⋅EtargetEcluster

To simulate a more realistic scenario, we also introduce noise to the energy profile. We use a daily factor αd (constant within each day) and an hourly factor βt:

αd&∼N(1,σd2), σd=0.08, αd∈[0.7,1.3]βt&∼N(1,σh2), σh=0.03, βt∈[0.9,1.1]

The synthetic series before re-normalization:

Ltsyn=Ltscaled⋅αd(t)⋅βt

Finally, as we did in the previous step, we re-scale the series to preserve the target annual energy:

Cb(t)=Ltsyn⋅Etargett=1TLtsyn.

The single-office profile has a mean load of 6.73  and a peak load of 11.92 .

#### Solar Photovoltaic System

The solar PV system is modeled by the following equation:

Psolar(t)=Apanel⋅ηpanel⋅I(t),

where:

- Apanel = solar panel area (m²)

- ηpanel = panel efficiency

- I(t) = solar irradiance at hour t (W/m²)

#### Building Battery Storage

The building’s battery is modeled as follows:

$$\begin{equation}
    P_{dis,t} = \min\!\left( P_{max}, \; (\soc_{t-1} - \soc_{min}) \cdot \eta \right)
    \label{eq:battery_discharge_power}
    \end{equation}$$

$$\begin{equation}
    P_{ch,t} = \min\!\left( P_{max}, \; C - \soc_{t-1} \right)
    \label{eq:battery_charge_power}
    \end{equation}$$

$$\begin{equation}
    \soc_t = \soc_{t-1} + \min\!\left( P_{ch,t}, \; E_{excess}(t) \right)
    \label{eq:soc_charge}
    \end{equation}$$

$$\begin{equation}
    \soc_t = \soc_{t-1} - \min\!\left( \frac{P_{dis,t}}{\eta}, \; \frac{E_{missing}(t)}{\eta} \right)
    \label{eq:soc_discharge}
    \end{equation}$$

where:

- Pmax = maximum charge/discharge power []

- C = battery capacity []

- $\soc_{min}$ = minimum allowable state of charge

- η = round-trip efficiency

- Eexcess(t)=max(0, Psolar(t)−Cb(t)) = surplus solar energy

- Emissing(t)=max(0, Cb(t)−Psolar(t)) = energy deficit

Constraints:

$$\begin{align}
    \soc_{min} &\leq \soc_t \leq C \\
    0 &\leq P_{ch,t} \leq P_{max} \\
    0 &\leq P_{dis,t} \leq P_{max}
    \end{align}$$

#### Net Energy Demand

The building’s net energy demand (positive = needs grid, negative = surplus):

Enetbuilding(t)=Cb(t)−Psolar(t)−Edischargeb(t)+Echargeb(t).

### Electric Vehicle Fleet Model

#### Individual EV Characteristics

Each EV i in the fleet has the following information:

- Battery capacity: Bcapi []

- State of charge: $\soc^i(t)$ [dimensionless, 0–1]

- Maximum charge rate: Pmax,chargei []

- Maximum discharge rate: Pmax,dischargei [] (for V2G)

- Arrival time: tarrivali [hour]

- Departure time: ttargeti [hour]

- Desired SoC: $\soc_{desired}^{i}$ [dimensionless]

#### EV Battery Dynamics

The state of charge evolves for EV i following the equation:

$$\begin{equation}
    \soc^i(t+1) = \soc^i(t) + \frac{\eta_{charge} \cdot E_{charge}^i(t) - E_{discharge}^i(t)/\eta_{discharge}}{B_{cap}^{i}}
    \end{equation}$$

where ηcharge and ηdischarge are charging and discharging efficiencies (typically 0.95).

#### Constraints

Each EV must satisfy:

$$\begin{align}
    \soc_{min} &\leq \soc^i(t) \leq 1.0 \quad \forall t \label{eq:soc_bounds} \\
    \soc^i(t_{target}^i) &\geq \soc_{desired}^i \label{eq:target_constraint} \\
    E_{charge}^i(t) &= 0 \quad \text{if } t < t_{arrival}^i \text{ or } t \geq t_{target}^i \label{eq:availability}
    \end{align}$$

Constraint [eq:target_constraint] ensures that each EV reaches its desired charge level before departure.

### Grid Model

#### Dynamic Pricing

For the simulations, we use dynamic pricing with a different price for each hour:

$$\begin{equation}
    \pi_{buy}(t) = \text{price\_profile}[t \bmod 24] \quad [\text{€/\kwh}].
    \end{equation}$$

For V2G scenarios, the selling prices are 80% of the buying prices:

$$\begin{equation}
    \pi_{sell}(t) = \text{sell\_price\_profile}[t \bmod 24] \quad [\text{€/\kwh}].
    \end{equation}$$

Typically, πsell(t)≤πbuy(t) (80-90% of purchase price).

#### Grid Capacity Constraint

The grid provides a certain capacity that can be used by the building for charging:

Egridbuilding(t)+i=1NEVEgridi(t)≤Pgrid,max(t),

where Pgrid,max(t) represents the hourly grid capacity limit.

## Optimization Problem Formulation

### Objective Function

The goal is to minimize the total electricity cost:

mint=0T−1πbuy(t)⋅Egridtotal(t)−πsell(t)⋅EV2Gtotal(t),

where:

- Egridtotal(t) = total energy purchased from grid at hour t

- EV2Gtotal(t) = total energy sold back to grid (V2G only)

- T = simulation duration (24 hours)

### Energy Balance

For each hour t, the energy equation must be satisfied:

Cb(t)+i=1NEVEchargei(t)=Psolar(t)+Egridtotal(t)+i=1NEVEdischargei(t)

## Level 0: Baseline (Uncontrolled Charging)

### Algorithm Description

The simple algorithm prioritizes the remaining solar energy and respects grid capacity; each car charges at the maximum rate upon arrival until its desired state of charge is reached:

Calculate building surplus: Esurplus(t)=max(0,Psolar(t)−Cb(t)) Calculate available grid capacity: Pavailable(t)=Pgrid,max(t)−max(0,Cb(t)−Psolar(t)) Eremaining←Esurplus(t) $E_{needed}^i = \min(P_{max,charge}^{i}, (\soc_{desired}^i - \soc^i(t)) \cdot B_{cap}^{i})$ Efrom_surplus=min(Eneededi,Eremaining) Efrom_grid=min(Eneededi−Efrom_surplus,Pavailable(t)) Etotal=Efrom_surplus+Efrom_grid Charge EV i with Etotal Update Eremaining and Pavailable(t).

### Mathematical Formulation

Energy for EV i at hour t:

$$\begin{equation}
    E_{charge}^i(t) = \min\left( P_{max,charge}^{i}, \, (\soc_{desired}^i - \soc^i(t)) \cdot B_{cap}^{i}, \, E_{available}(t) \right)
    \end{equation}$$

where Eavailable(t) contains both the building’s surplus energy and the remaining grid capacity.

## Level 2: Intelligent Optimization Algorithms

### Reinforcement Learning (Q-Learning)

#### State Space

The state for EV i at hour t is represented by the following vector:

$$\begin{equation}
    s^i(t) = (\text{EV\_id}, \, \soc_{bin}^i(t), \, t, \, \text{grid\_availability}),
    \end{equation}$$

where $\soc_{bin}^i(t)$ is the discretized SoC (bins of 0.1).

#### Action Space

For charge-only systems:

A={charge,standby}.

For V2G-enabled systems:

AV2G={charge,discharge,standby}.

#### Reward Function

The reward function is as follows:

$$\begin{align}
    R(s,a,s') = &\, -\alpha_1 \cdot E_{grid}^i(t) \cdot \pi_{buy}(t) \quad \text{(minimize grid cost)} \nonumber \\
                &\, +\alpha_2 \cdot E_{solar}^i(t) \quad \text{(maximize solar usage)} \nonumber \\
                &\, +\alpha_3 \cdot \mathbb{1}[\soc^i(t_{target}^i) \geq \soc_{desired}^i] \quad \text{(target achievement)} \nonumber \\
                &\, -\alpha_4 \cdot \max(0, \soc_{desired}^i - \soc^i(t_{target}^i)) \quad \text{(penalty for shortfall)} \nonumber \\
                &\, -\alpha_5 \cdot \mathbb{1}[\soc^i(t) < \soc_{min}] \quad \text{(constraint violation)}
    \end{align}$$

where α1,…,α5 are weights.

#### Q-Learning Update Rule

The Q-value is updated using the temporal difference method:

Q(s,a)←Q(s,a)+αR(s,a,s′)+γmaxa′Q(s′,a′)−Q(s,a),

where:

- α = learning rate (0.1-0.15)

- γ = discount factor (0.95-1.0)

- s′ = next state after taking action a in state s

#### Policy Execution

After training, the policy selects actions greedily:

a*(s)=argmaxa∈AQ(s,a).

## Level 2: Vehicle-to-Grid (V2G)

### Bidirectional Energy Flow

V2G expands the optimization to include discharge decisions:

Eneti(t)=Echargei(t)−Edischargei(t).

### V2G Constraints

Additional constraints for V2G are as follows:

$$\begin{align}
    E_{discharge}^i(t) &\leq (\soc^i(t) - \soc_{desired}^i) \cdot B_{cap}^{i} \quad \text{(preserve target SoC)} \\
    E_{charge}^i(t) \cdot E_{discharge}^i(t) &= 0 \quad \text{(no simultaneous charge/discharge)}.
    \end{align}$$

### Revenue Calculation

Total benefit with V2G:

BenefitV2G=−t=0T−1πbuy(t)⋅Egridtotal(t)−πsell(t)⋅i=1NEVEdischargei(t).

Positive benefit indicates revenue or cost saving.

### V2G Integration with Algorithms

The Q-Learning algorithm is extended to handle V2G by adding the action of discharging and by adding the selling reward to the reward function; additionally, discharge constraints are included.