# Data Tables and Additional Results

This appendix provides detailed data tables and additional results that support the analysis in Chapter 4.

## Configuration Parameters

### Complete Building Parameters

Complete building configuration parameters

| Parameter | Value | Unit |
| --- | --- | --- |
| Solar PV System (JA Solar DeepBlue 3.0 Pro ): |  |  |
| Reference module | JAM54S31-405/MR | — |
| Module rated power | 405 | Wp |
| Module efficiency (STC) | 20.7 | % |
| Cell type | Mono PERC half-cut | — |
| Module dimensions | 1722 × 1134 × 30 | mm |
| Number of modules | 74 | — |
| Total panel area | ≈ 150 | m² |
| System efficiency (sim.) | 20 | % |
| Peak solar irradiance | 1000 | W/m² |
| Peak power output | 30 | kW |
| Building Battery (BYD Battery-Box Premium HVM ): |  |  |
| Chemistry | LiFePO4 (LFP) | — |
| Module capacity | 2.76 | kWh |
| Configuration | 4 × HVM 19.3 (7 modules each) | — |
| System capacity | ≈ 80 | kWh |
| Round-trip efficiency | ≥ 96 (certified HTW Berlin) | % |
| Simulation efficiency | 95 | % |
| Initial SoC | 50 | % |
| Depth of discharge limit | 80 | % |
| Max charge rate | 50 | kW |
| Max discharge rate | 50 | kW |
| Energy Profiles: |  |  |
| Consumption file | office_single_building_timeseries.csv | — |
| Irradiance file | irradiance_pvgis.csv | — |
| Simulation date | March 12, 2018 | — |

### Complete EV Fleet Parameters

Complete EV fleet configuration parameters

| Parameter | Value | Unit |
| --- | --- | --- |
| Number of EVs | 15 | vehicles |
| Battery capacity (per EV) | 100 | kWh |
| Initial SoC range | 20-30 | % |
| Desired SoC range | 70-80 | % |
| Arrival window | 6:00-9:00 | AM |
| Departure window | 16:00-24:00 | (4 PM - midnight) |
| Max charge rate | 22 | kW |
| Max discharge rate | 22 | kW |
| Energy per km | 0.18 | kWh/km |
| Depth of discharge limit | 90 | % |
| Minimum SoC | 20 | % |

## Hourly Data Tables

### Electricity Price Profile

Table 1.3 presents the complete hourly day-ahead electricity prices for the French bidding zone on March 12, 2018, as retrieved from the ENTSO-E Transparency Platform  and cross-validated via the Fraunhofer ISE Energy-Charts API . Sell-back prices for the V2G scenario are set at 80% of the purchase price.

Hourly day-ahead electricity prices — France (EPEX SPOT), March 12, 2018

| Hour | EUR/MWh | Purchase (€/kWh) | Sell (€/kWh) | Spread (%) |
| --- | --- | --- | --- | --- |
| 00:00 | 28.46 | 0.0285 | 0.0228 | 80% |
| 01:00 | 19.80 | 0.0198 | 0.0158 | 80% |
| 02:00 | 12.00 | 0.0120 | 0.0096 | 80% |
| 03:00 | 9.56 | 0.0096 | 0.0076 | 80% |
| 04:00 | 19.04 | 0.0190 | 0.0152 | 80% |
| 05:00 | 42.01 | 0.0420 | 0.0336 | 80% |
| 06:00 | 44.66 | 0.0447 | 0.0357 | 80% |
| 07:00 | 48.34 | 0.0483 | 0.0387 | 80% |
| 08:00 | 48.05 | 0.0481 | 0.0384 | 80% |
| 09:00 | 46.02 | 0.0460 | 0.0368 | 80% |
| 10:00 | 44.41 | 0.0444 | 0.0355 | 80% |
| 11:00 | 45.00 | 0.0450 | 0.0360 | 80% |
| 12:00 | 41.50 | 0.0415 | 0.0332 | 80% |
| 13:00 | 38.38 | 0.0384 | 0.0307 | 80% |
| 14:00 | 37.24 | 0.0372 | 0.0298 | 80% |
| 15:00 | 37.61 | 0.0376 | 0.0301 | 80% |
| 16:00 | 42.23 | 0.0422 | 0.0338 | 80% |
| 17:00 | 50.24 | 0.0502 | 0.0402 | 80% |
| 18:00 | 62.00 | 0.0620 | 0.0496 | 80% |
| 19:00 | 55.23 | 0.0552 | 0.0442 | 80% |
| 20:00 | 43.23 | 0.0432 | 0.0346 | 80% |
| 21:00 | 41.23 | 0.0412 | 0.0330 | 80% |
| 22:00 | 36.81 | 0.0368 | 0.0294 | 80% |
| 23:00 | 35.04 | 0.0350 | 0.0280 | 80% |
| Average | 38.67 | 0.0387 | 0.0309 | 80% |

### Building Energy Profile

Hourly building consumption and solar production

| Hour | Consumption (kW) | Solar Production (kW) | Net Demand (kW) |
| --- | --- | --- | --- |
| 00:00 | 6.49 | 0.0 | 6.49 |
| 01:00 | 6.62 | 0.0 | 6.62 |
| 02:00 | 6.39 | 0.0 | 6.39 |
| 03:00 | 6.54 | 0.0 | 6.54 |
| 04:00 | 6.41 | 0.0 | 6.41 |
| 05:00 | 7.47 | 0.0 | 7.47 |
| 06:00 | 7.67 | 0.0 | 7.67 |
| 07:00 | 8.36 | 1.25 | 7.11 |
| 08:00 | 9.74 | 5.05 | 4.69 |
| 09:00 | 9.62 | 5.89 | 3.73 |
| 10:00 | 9.33 | 2.09 | 7.24 |
| 11:00 | 9.68 | 6.38 | 3.30 |
| 12:00 | 9.14 | 5.61 | 3.53 |
| 13:00 | 9.75 | 7.24 | 2.51 |
| 14:00 | 8.98 | 3.96 | 5.02 |
| 15:00 | 9.09 | 14.62 | −5.53 |
| 16:00 | 8.84 | 5.86 | 2.98 |
| 17:00 | 8.41 | 0.80 | 7.61 |
| 18:00 | 8.02 | 0.0 | 8.02 |
| 19:00 | 8.21 | 0.0 | 8.21 |
| 20:00 | 7.29 | 0.0 | 7.29 |
| 21:00 | 6.62 | 0.0 | 6.62 |
| 22:00 | 6.81 | 0.0 | 6.81 |
| 23:00 | 6.20 | 0.0 | 6.20 |
| Total | 191.7 | 58.8 | — |

Note: Negative net demand indicates surplus solar energy available for EV charging.

## Individual EV Trajectories

### EV State of Charge Evolution

Individual EV SoC trajectories (RL method, charge-only)

| Hour | EV 1 | EV 2 | EV 3 | ... | EV 15 | Average |
| --- | --- | --- | --- | --- | --- | --- |
| 00:00 | 25% | 22% | 28% | ... | 24% | 25% |
| 06:00 | 25% | 22% | 28% | ... | 24% | 25% |
| 07:00 | 30% | 28% | 33% | ... | 29% | 30% |
| 08:00 | 38% | 35% | 41% | ... | 37% | 38% |
| 09:00 | 46% | 43% | 49% | ... | 45% | 46% |
| 10:00 | 54% | 51% | 57% | ... | 53% | 54% |
| 11:00 | 62% | 59% | 65% | ... | 61% | 62% |
| 12:00 | 70% | 67% | 73% | ... | 69% | 70% |
| 13:00 | 75% | 72% | 78% | ... | 74% | 75% |
| 14:00 | 76% | 73% | 79% | ... | 75% | 76% |
| 15:00 | 76% | 73% | 79% | ... | 75% | 76% |
| 16:00 | 76% | 73% | 79% | ... | 75% | 76% |